In vitro experiment on animal heart using Langendorff apparatus

(Pre- and post-test)

- With which equipment, heart reservoir is connected with 3/8” tubing?

- Blood reservoir

- Heater/oxygenator

- Centrifugal pump

- Hemostatic valve

- Why washing of blood with normal is recommended before using in this   experiment?

- To remove excess electrolyte

- To remove lysed cellular materials

- To remove red blood cell

- All the above

- Which of the following hematocrit concentration is recommended for using in Langendorff apparatus?

- 20-25%

- 30-35%

- 40-45%

- 50-55%

- What should be the initial calcium ion concentration in the blood during experiment with Langendorff apparatus?

- 0.3-0.5 mmol/L

- 0.5-0.7 mmol/L

- 0.7-0.9 mmol/L

- 0.9-0.11 mmol/L

- What parameters do you need to check during this experiment?

  - Blood pH

  - Blood electrolyte concentration

  - Temperature

  - Clotting factor

- How often do you have to take blood sample to check physiologic parameter?

  - Every 5 minute interval

  - Every 10 minute interval

  - Every 15 minute interval

  - Every 20 minute interval

- What amount of ionic calcium do you have to add frequently in the blood during the experiment?

  - 1 mmol

  - 2 mmol

  - 3 mmol

  - 4 mmol

- How will you insert the pressure transducer?

  - Through hemostasis valve into the aorta

  - Through hemostasis valve into the atrium

  - Through hemostasis valve into the ventricle

  - Through hemostasis valve into the septum

- Temperature of warming unit should be increased until intramyocardial temperature is measured at 37⁰ C. T/F

- Pressure-volume conductance catheter is inserted in to the left ventricle through apical incision. T/F